Municipality of Alleyn and Cawood

- - Construction By-Law-

No. 07-039

Revision to comply with the Regional Schéma of the MRC of Pontiac –Art. 59 LAU -

P.U.R.E

Planification.Urbanisme.Rural.Environnement.

Consultant en urbanisme

Table Of Contents

1.0	Declaratory and interpretative provisions	3

1.1	Title of the By-Law	3

1.2	Objective	3

1.3	Persons affected	3

1.4	Abrogation of previous By-Laws	3

1.5	Persons affected	3

1.6	Amendments	4

1.7	Partial invalidity of the By-Law	4

1.8	The By-Law  and the laws	4

2.0	Materials and assemblies	5

2.1	Technical documents	5

2.2	Material used on external finish	5

2.3	Fondations	5

2.4	Maintenance of buildings	5

2.5	Access to dwelling	6

2.6	Radio, radar and television antennas	6

2.7	Forbidden constructions	6

3.0	Provisions applicable to mobile homes	7

3.1	Height of foundation	7

3.2	Belt of technical space	7

3.3	Steps	7

3.4	Construction	7

3.5	Annexes	7

3.6	Platform	7

3.7	supports	7

3.8	Anchors	8

4.0	Miscellaneous provisions	9

4.1	Foundations	9

4.2	Requirement for fire-proof party walls	9

4.3	Snow and ice accumulation	9

4.4	Vacant or incomplete construction	9

4.5	Septic installation	9

4.6	provisions concerning construction on islands	10

On public land	10

On private land	10

4.7	Building destroyed by fire	10

5.0	Abrogatives provisions	11

6.0	Effective date	12

# Declaratory and interpretative provisions

## Title of the By-Law

The title of this By-law is «By-law concerning construction no 07-039 » of the Municipality of Alleyn and Cawood.

The By-law titled “Administration and Interpretation of the Planning By-laws No 07-039 “ regulates the present By-law.

(Same article as By-law  #1591,but clearly reworded)

## Objective

The objective of this By-law is to safeguard life, prevent bodily harm, maintain sanitary condition and protect the property and the public, through the enforcement of minimum standards for the design, the construction and the alterations of buildings.

(Integrally copied from By-law #1591)

## Persons affected

This By-law, which provisions aim any individual and to any legal entity, public or civil, applies to the whole of the territory under this jurisdiction of the Municipality of Alleyn and Cawood.

(Same article as By-law  #1591,but clearly reworded)

## Abrogation of previous By-Laws

The By-law #1591, its amendments and any sketches, which are included, are abrogated and replaced by the present By-law.

Are also abrogated, all other regulatory provisions incompatible with the present Construction By-law.

Such replacements do not however affect procedures instituted under the authority of the By-laws hereby replaced, of which will proceed under the authority of the said replaced By-laws until final judgment and execution. Also, they do not affect the permits delivered under the authority of the By-laws hereby replaced.

(Same article as By-law  #1591, but clearly reworded.)

## Persons affected

This by-law applies to any legal entity, public or civil, and to any individual.

(Integrally copied from By-law #1591)

## Amendments

The provisions of this By-law cannot be adopted, modified or abrogated but by an approved By-law, in accordance with the applicable provisions of the Provincial Planning Act.

(Same article as By-law  #1491, but clearly reworded.)

## Partial invalidity of the By-Law

Annulment by the Court, in whole or in part, of one or more of the articles of this by-law, does not result in the annulment of the whole by-law, this by-law having been adopted word-by-word, article-by-article. In the case where a part, a clause or a provision of this by-law should be declared invalid of all other parts, clauses or provisions would not be under challenge.

The municipal Council hereby declares that it would have decreed what remains of this by-law even if one or more clauses are declared invalid.

(Integrally copied from By-law #1591)

## The By-Law  and the laws

No article of this By-law can shield anyone from the application of a law of Canada or of Québec.

(Integrally copied from By-law #1591)

# Materials and assemblies

## Technical documents

The National Building Code, the Québec Plumbing Code, the Code for Québec Electricians, the Safety Code for Construction work (Québec), the Fire Prevention Code for Canada, are used as reference documents to apply this By-law.

All provisions of the Québec Building Code, of the National Building Code for Canada, of the National Fire Prevention Code for Canada, in force on the date of a request for a construction permit, which are not incompatible with the provisions of this By-law, must be respected, as much as possible.

Amendments made to these technical documents, following the coming into force of these By-laws, also become reference documents to this construction by-law. These amendments come into force in the municipality on the date decided by resolution of the Municipal Council. The Secretary-treasurer must give public notice of the adoption of this resolution.

(From By-law #1591)

## Material used on external finish

The following materials are forbidden on visible exterior walls:

-- asphalt shingles

-- mineralized or tarred paper or rolls

-- reflecting materials on roofs or on front walls.

(Integrally copied from By-law #1591)

## Fondations

Any main Building must rest on continuous foundations made of stone, concrete or cement blocks, or on other approved materials. No main construction may be erected on stone, concrete, brick or wooden pillars.

Seasonal dwellings, intended for summer occupation only, may be erected on stone, concrete, brick or wooden pillars, providing that spaces between pillars are closed with latticework, or by another material approved by the inspector.

Small garages and carports must conform with section 9-36 of the National Building Code.

(Integrally copied from By-law #1591)

## Maintenance of buildings

Main Buildings and annexes must be kept in good order, repaired as required and painted when necessary.

(Integrally copied from By-law #1591)

## Access to dwelling

Each dwelling in a collective residence must be accessible without going through another dwelling.

(Integrally copied from By-law #1591)

## Radio, radar and television antennas

Radio, radar and television antennas must be erected in a way that will not spoil the appearance of the building on which they are installed, as well as the surroundings.

(Integrally copied from By-law #1591)

## Forbidden constructions

The use of railway cars, tramways, busses or other unused vehicles of the same nature is forbidden for all purposes. Only trailers or commercial use motor vehicles are allowed, but only on construction sites.

(Integrally copied from By-law #1591)

# Provisions applicable to mobile homes

## Height of foundation

Any kind of foundation on which a mobile home is erected must not be higher than 1.0 meter.

(Integrally copied from By-law #1591)

## Belt of technical space

All mobile homes must be provided with a belt of mechanical space between the bottom part of the unit and the ground, and equipped with a mobile trap measuring 1.0 meter wide by 60 centimetres high to allow access to public services hook-ups. The finish of that belt of mechanical space must include a protective coating.

(Integrally copied from By-law #1591)

## Steps

All mobile homes must be equipped with steps, landing and hand-rails leading to each entrance, in accordance with the Canadian Code for Residential Constructions (Residential Standards 1970). Steps must be at least 1.0 meter wide and be painted if they are not of concrete or aluminium.

(Integrally copied from By-law #1591)

## Construction

All annexes and auxiliary constructions must be erected in accordance with the requirements of the construction by-law. All annexes or auxiliary constructions such as porches, solariums, belts for mechanical space as well as storage sheds, must be prefabricated or of equivalent quality, and must be painted or pre-finished to ensure their model and construction are in harmony with the main construction.

(Integrally copied from By-law #1591)

## Annexes

Annexed constructions must not obstruct the required open spaces for the lighting and the ventilation of the mobile home, nor prevent the inspection of the equipment of the mobile home, or the hook-ups to public services, nor encroach on the 2 meters side court required.

(Integrally copied from By-law #1591)

## Platform

A platform must be installed on the lot for each mobile home, and designated to support evenly the maximum anticipated load of a mobile home in any season, without there being any sinking-in nor any other kind of movement.

(Integrally copied from By-law #1591)

## supports

On that platform the mobile home must be supported and held with the help of pillars, posts or other acceptable means, installed at a sufficient depth to prevent any movement caused by frost, and in order to support the anticipated load at the points on the chassis indicated by the manufacturer, or established in accordance with the CSA Standards for mobile Home construction 2240.2 (1970). Particular care must be taken to ensure that in the case of an extensible unit, all parts of the mobile home are adequately supported.

(Integrally copied from By-law #1591)

## Anchors

Anchors, in the shape of metal eyelets embedded in poured concrete, of corkscrews or of arrowhead anchors, must be provided at all angles of the platform for the mobile home, and at such places where they may be required to solidly attach the mobile home, and allow it to withstand windy conditions. The mobile home chassis anchoring devices must be attached with a cable or other approved device. The anchor itself and the means to secure it, must be able to withstand a tension of at least 2180 kilograms (4,800 lbs).

(Integrally copied from By-law #1591)

# Miscellaneous provisions

## Foundations

All foundations must be waterproof and set at a depth protected from freezing. The thickness of foundation walls must be at least equal to that of the walls they support.

(Integrally copied from By-law #1591)

## Requirement for fire-proof party walls

When two buildings are contiguous, they must be separated by a fireproof party wall built in stone, concrete, cement blocks or solid brick.

(Integrally copied from By-law #1591)

## Snow and ice accumulation

Exterior walls and roof sheathing of main buildings and annexes must be perfectly water-tight, and roofs kept free at all times, of any accumulation of snow or ice that could constitute a danger for the public.

Council declares that to let snow and ice accumulate on roofs where it could be a public danger, constitutes a nuisance within the terms of the Law.

(Integrally copied from By-law #1591)

## Vacant or incomplete construction

All vacant or incomplete construction must be properly closed and boarded in order to prevent any accident.

(Integrally copied from By-law #1591)

## Septic installation

All septic installation must conform to the requirements of the « Appropriate Québec Ministry », Decree No. 1886-81 of July 9th, 1981, in force on August 12th, 1981.

(Integrally copied from By-law #1591)

## provisions concerning construction on islands

The following provisions apply to all the islands unconnected to the mainland by a bridge or a culvert. These provisions aim at the preservation of these fragile environments.

### On public land

Construction of any new dwelling is forbidden on islands of public holding, unless it is for the purposes of an outfitter or their utilization for recreational purposes.

(Integrally copied from MRC of Pontiac - Revised Land Development Plan, By-Law Number 65-99, The Complementary Document)

### On private land

On islands of private holding, the following provisions do apply:

- When a construction project provides for a water supply system with pipes under pressure, a study made by an engineer member of the Québec Order of Engineers (OIQ) must show that the piece of land is suitable for septic tanks. The application must also include an agreement to service the septic tank. The minimal area of any lot is 5,600 m2 ;

- Independently of the slope, the waterside protection strip is at least 25 metres.

(Integrally copied from MRC of Pontiac - Revised Land Development Plan, By-Law Number 65-99, The Complementary Document)

## Building destroyed by fire

Any construction or repair of any building destroyed or made dangerous, or having lost seventy-five percent (75%) of its value on the assessment roll following a fire or any other cause, must be completed in conformity with the B-laws in force at the time of that reconstruction or repair.

(Integrally copied from By-law #1591)

# Abrogatives provisions

The current By-law abrogates all relative regulatory descriptions as well as By-law N0 1591 and its modifications.

# Effective date

The current By-law will come into effect once all necessary procedures under the law have been fulfilled.

Adoption of the Draft By-law

GIVEN AT ALLEYN ET CAWOOD, QUÉBEC this 3th day of the month of December 2007.

................................................... ..................		.......................................................................

Joseph Squitti					Kim Cartier-Villeneuve,

Mayor						Secretary-treasurer

Adoption date of notice of motion:		01 October 2007

Adoption date of Draft By-law:			01 October 2007

Adoption date of By-law:			03 December 2007

Reception date of the conformity certificate: